Recognize further

Endure longer

S917V10 with UHF module

Rugged Tablet PC

1

UHF module in V10

- Operating frequency band: 2.45GHz

- Chip: NRF52832

- Identification distance: 0-50m (adjustable)

- Recognition speed: 200km/h

- Antenna interface IPEX, external antenna can be connected

- Using hybrid data acquisition anti-collision algorithm

- Support active mode and passive mode

- Support real-time upload mode (default), scheduled upload mode and quantitative upload mode

Functional characteristics of UHF module

Serial port active module

This product is developed by a group of senior engineers with more than 10 years of rich working experience. It is a simple, easy-to-use, stable and cost-effective equipment.

It can be quickly integrated, and is built-in installed on handheld phones, Android tablets, electronic class cards, and attendance machines. It is widely used in pet and item tracking, mobile asset inventory, campus attendance, class check-in, etc.

1

- Asset management —— Handset inventory application

- Smart attendance —— Student attendance and class attendance

- smart building —— Indoor personnel positioning management and visitor positioning management

Typical applications

Electrical characteristics

Operation reference conditions：

| parameter | minimum value | Typical value | Maximum value | unit |
| --- | --- | --- | --- | --- |
| input voltage | 3 | 3.3 | 3.6 | V |
| working temperature | -20 | 25 | 70 | ℃ |
| RF parameters： storage temperature | -40 | 25 | 85 | ℃ |

| parameter | minimum value | Typical value | Maximum value | unit |
| --- | --- | --- | --- | --- |
| transmit power | -20 | --- | 4.0 | dBm |
| receiving sensitivity | -89 | -93 | -96 | dBm |
| working frequency | 2360 | -- | 2500 | MHz |
| Basic parameters of module communication rate | 250K | 1M | 2M | bit/s |

| working frequency | 2360MHz~2500MHz |  |
| --- | --- | --- |
| working voltage | 3.3V |  |
| Maximum transmit power | +20dbm |  |
| Wireless rate | 1Mbps/2Mbps |  |
| Emission current | About 95mA@20dbm output |  |
| Receive current | About 35mA@2Mbps |  |
| MCU Working current | 4.4mA@32MHz@3V | Run from flash |
| MCU Idle mode | 2.3mA@3V on mode |  |
| MCU Off mode | 420uA@3V off mode |  |
| Typical sensitivity | -96dbm@250kbps |  |
| Pin spacing | 1.27mm |  |

Website: https://www.senter-e.com/

Linked-in: Shandong Senter Electronic Co, Ltd

Add: No.18 Liuyishan Road,New&High-tech Area,Zibo City, Shandong Province, China

Tel: 0533-3589257

E-mail:export@senter.com.cn

About Senter